SERI N.S SDN. BHD.

# Complete Drone Operations Manual

Joyance JTC30 Façade Glass and Solar Panel Cleaning

| Document Control | Details |
| --- | --- |
| Document Number | SNS-UAS-OM-001 |
| Revision | 02 |
| Effective Date | ________________ |
| Prepared By | ________________ |
| Reviewed By | ________________ |
| Approved By | ________________ |
| Controlled Copy | ________________ |

CONTROLLED DOCUMENT

This manual establishes company procedures but does not grant authority to fly. Every operation requires valid approvals and must remain within the aircraft manufacturer limits and site-specific controls.

# Manual Status and Safety Principle

This manual governs planning, mobilization, operation, cleaning, emergency response, maintenance and reporting for the company’s drone-cleaning activities. The remote pilot in command retains final authority over launch, continuation and landing. Any crew member may call STOP when an unsafe condition exists.

No commercial deadline, client request or cost consideration overrides an approval condition or safety limit. Where this manual, the CAAM approval, the manufacturer manual, a site rule, an SDS or the project risk assessment differ, the most restrictive requirement applies.

## Regulatory Release Gate

☐  CAAM approval is valid for the exact site, dates, aircraft, remote pilot and operating area.

☐  The approval expressly covers the water, detergent, hose or other cleaning payload activity.

☐  Night work is undertaken only when the approval expressly permits the approved night period and conditions.

☐  Airspace, landowner, building-management, police, local-authority, security and radio approvals required for the site are valid.

☐  The site-specific ConOps, risk assessment, method statement and emergency response plan are approved and briefed.

If any item is missing, unclear, expired or inconsistent with the planned work, the operation is postponed and referred to the accountable manager.

## Revision Record

| Rev | Date | Description | Approved By |
| --- | --- | --- | --- |
| 00 | ________ | Initial complete operations manual | ________ |
| 01 | ________ | Added verified JTC30 aircraft, controller, pump, charger and battery technical details from JTC30 Tech Spec v01 | ________ |
| 02 | ________ | Added Midwest van-mounted cleaning and pure-water supply system details, procedures, inspection and maintenance controls | ________ |

## Controlled Distribution

| Copy | Holder or Location | Format | Status |
| --- | --- | --- | --- |
| 01 | Accountable Manager | Master electronic | Controlled |
| 02 | Operations Manager | Electronic | Controlled |
| 03 | Remote Pilot Team | Field copy | Controlled |

# Contents

1 Purpose Scope and Operating Policy

2 Definitions and Abbreviations

3 Regulatory and Client Compliance

4 Organization Roles and Responsibilities

5 Competency Fitness and Training

6 Aircraft Cleaning System and Support Equipment

7 Operational Control and Configuration Management

8 Customer Order and Project Lifecycle

9 Site Survey and Feasibility Assessment

10 Concept of Operations and Flight Planning

11 Risk Assessment and Work Authorization

12 Weather and Environmental Conditions

13 Ground Safety and Exclusion Zones

14 Mobilization and Site Setup

15 Pre Flight Inspection and Briefing

16 Standard Daytime Flight Operations

17 Façade and Glass Cleaning Operations

18 Solar Panel Cleaning Operations

19 Water Detergent and Runoff Controls

20 Night Operations

21 Abnormal and Emergency Procedures

22 Incident Near Miss and Damage Reporting

23 Post Flight Demobilization and Customer Reporting

24 Maintenance Inspection and Defect Control

25 Battery Charging Transport and Storage

26 Records Audit and Continuous Improvement

Appendices Field Checklists and Forms

# 1 Purpose Scope and Operating Policy

## 1.1 Purpose

The manual provides a single, repeatable operating system for safe and compliant drone cleaning from customer order to final report. It is intended for managers, remote pilots, visual observers, pump operators, technicians, safety marshals and supporting personnel.

## 1.2 Scope

It covers external cleaning of building façades, painted surfaces, glass, cladding, signs and solar panels with the Joyance JTC30 in an approved configuration. It includes daytime and approved night operations, water and detergent delivery, hose management, ground control, inspections, records and emergency response.

## 1.3 Exclusions

BVLOS unless separately assessed and specifically approved.

Flight over uninvolved persons, public roads or uncontrolled areas.

Indoor, confined-space, explosive-atmosphere or hazardous-area flight unless separately engineered and approved.

Work near exposed live electrical conductors without verified isolation and specialist controls.

Lifting, carriage or dispensing outside the approved cleaning configuration.

Any use beyond manufacturer, approval, insurance or site limits.

## 1.4 Operating Policy

Operations use a plan brief execute monitor close approach. Every project has named authority, defined boundaries, measurable limits, documented release gates and a clear stop-work process.

# 2 Definitions and Abbreviations

| Term | Meaning |
| --- | --- |
| Accountable Manager | Person responsible for resources, compliance and organizational safety. |
| AGL | Above ground level. |
| ATC | Air traffic control. |
| CAAM | Civil Aviation Authority of Malaysia. |
| C2 Link | Command and control link between aircraft and remote pilot station. |
| ConOps | Concept of Operations describing the intended activity and controls. |
| EVLOS | Extended visual line of sight under the arrangement authorized by the approval. |
| Ground Risk Area | Area exposed to potential aircraft, hose, water, chemical or falling-object hazards. |
| Night | The period stated in the applicable CAAM rules or approval; CAD 6011 Part V defines it as 20 minutes after sunset to 20 minutes before sunrise at surface level. |
| PIC | Remote Pilot in Command. |
| SDS | Safety Data Sheet. |
| SORA | Specific Operations Risk Assessment. |
| UAS | Unmanned Aircraft System. |
| VLOS | Continuous unaided visual contact sufficient to control the flight path and avoid aircraft, people and obstacles. |
| Visual Observer | Trained person assisting the pilot by maintaining visual observation and hazard scan. |

# 3 Regulatory and Client Compliance

## 3.1 Approval Framework

UAS operations in Malaysia are subject to the Malaysian Civil Aviation Regulations and applicable CAAM Civil Aviation Directives. The CAAM UAS portal identifies approval pathways according to risk. Its Standard ATF description states that low-risk operations involve VLOS or EVLOS, no spraying or dispensation, and operation below 400 feet AGL unless otherwise approved by ATC. Drone cleaning therefore shall not be assumed to qualify for the ordinary low-risk pathway.

The accountable manager shall obtain written confirmation of the applicable approval route and ensure the issued approval expressly covers the intended cleaning system. Operations presenting additional risk may require the Special UAS Project process, including a detailed ConOps and risk assessment.

## 3.2 Approval Register

| Approval or Evidence | Owner | Verification Before Work |
| --- | --- | --- |
| CAAM approval and conditions | Accountable Manager | Reference, aircraft, pilot, site, dates, times, height, payload and limitations match. |
| Airspace or ATC coordination | Operations Manager | Required communications, notification and response actions are briefed. |
| Aircraft and radio equipment compliance | Technical Manager | Required certification or approvals are valid for installed equipment. |
| Landowner and client authorization | Project Manager | Written permission covers access, cleaning, water use, barriers, photos and working hours. |
| Building management permit | Project Manager | Loading, access, lift, water, drainage, security and after-hours rules confirmed. |
| Local authority police or security coordination | Project Manager | Any required road, crowd, security or photography controls completed. |
| Insurance | Accountable Manager | Aircraft, public liability, property damage, payload, night work and all teams covered. |

## 3.3 Privacy and Information Control

Photography and video shall be limited to operational need and client authorization. Cameras must not intentionally capture private interiors, unrelated persons or restricted information. Records shall be stored with controlled access and released only to authorized recipients.

## 3.4 Legal Review

The compliance register shall be reviewed before each new project type and at least annually. This manual must be amended when an approval, law, manufacturer limitation or operating configuration changes.

# 4 Organization Roles and Responsibilities

| Role | Authority and Duties |
| --- | --- |
| Accountable Manager | Provides resources; confirms approval, insurance and organizational compliance; approves manual and major changes. |
| Operations Manager | Controls scheduling, approval register, project release, weather review, crew assignment and operational records. |
| Project or Site Manager | Coordinates customer, permits, access, work sequence, site facilities, barriers and final acceptance. |
| Remote Pilot in Command | Makes go or no-go decision; operates aircraft; maintains limits; commands pump activity; initiates emergency action. |
| Visual Observer | Maintains continuous visual and airspace scan; monitors obstacles, people and hose; communicates clear alerts; performs no distracting task. |
| Cleaning System Operator | Controls pump, pressure, flow, detergent and emergency isolation only as directed by PIC. |
| Ground Safety Marshal | Maintains exclusion zone and controls pedestrians and vehicles; stops work on intrusion. |
| Technical Manager or Technician | Controls configuration, maintenance, defect assessment, serviceability and return-to-service release. |
| Safety Coordinator | Supports risk assessment, briefings, incident investigation, corrective actions and audit. |
| All Crew | Attend briefing, use PPE, remain fit for duty, report hazards and exercise stop-work authority. |

## 4.1 Minimum Manning

A routine cleaning flight shall have at least one PIC, one dedicated visual observer, one cleaning-system operator and sufficient safety marshals to control the full ground-risk area. Roles may not be combined where doing so creates distraction or conflicts with approval conditions.

# 5 Competency Fitness and Training

## 5.1 Competency

PIC holds the competency required by the approval and is current on the JTC30 and payload configuration.

Visual observer is trained in airspace scan, orientation, standard calls, hose hazards and emergency actions.

Pump operator is trained on pressure, flow, isolation, hose restraint, detergent and SDS requirements.

Technician is authorized for assigned inspection and maintenance tasks.

Crew completes site induction and task-specific briefing.

## 5.2 Initial and Recurrent Training

| Module | Initial Evidence | Recurrent or Trigger |
| --- | --- | --- |
| Regulatory and approval conditions | Assessment and briefing record | Annual and on regulatory change |
| JTC30 systems and limitations | Manufacturer/operator training | Annual and after major configuration change |
| Normal flight and cleaning passes | Supervised practical assessment | Six-month proficiency check |
| Emergency procedures | Scenario drills | Six-month drills and after incident |
| Night operations | Supervised night familiarization | Six-month night recency or re-familiarization |
| Chemical and spill response | SDS and practical briefing | Annual and when product changes |
| First aid fire and electrical awareness | Recognized training where required | Per certificate and company schedule |

## 5.3 Fitness and Fatigue

Safety-critical personnel shall declare fitness before duty. No person may operate while impaired by fatigue, illness, alcohol, drugs or medication. The operations manager shall plan rest, travel and night shifts conservatively and replace any person showing reduced alertness.

| Fatigue Trigger | Required Action |
| --- | --- |
| Extended travel before shift | Reassign or provide rest before safety-critical duty. |
| Repeated night shifts | Use approved roster and monitor cumulative fatigue. |
| Loss of concentration or repeated communication error | Pause operation and assess fitness. |
| Crew self-declaration | Accept without pressure; replace or postpone. |
| Adverse heat stress or dehydration | Rest, hydrate and reassess before duty. |

# 6 Aircraft Cleaning System and Support Equipment

## 6.1 Approved System

The approved system includes the JTC30 aircraft, remote controller, software, navigation and anti-collision lights, batteries, charger, payload mount, nozzle, hose, strain relief, pump, filter, water source, detergent system and all communications and ground-safety equipment. Removing or adding a component may change mass, centre of gravity, performance and approval basis.

## 6.2 JTC30 Technical Details

The following values are transcribed from JTC30 Tech Spec v01 supplied for the company aircraft. They support planning but do not replace the current manufacturer manual, issued approval, aircraft configuration record or site-specific risk assessment.

| System | Confirmed Specification |
| --- | --- |
| Aircraft model | JOYANCE JTC30 cleaning drone |
| Protection | Overall protection level stated as IP55; described as dustproof, waterproof and corrosion-resistant. |
| Flight-control model | VKAG V9 |
| Included modules | FMU, PMU, GPS, RTK, LED and CANHUB |
| Positioning | GPS using BDS, GPS and GLONASS; RTK supported |
| Flight-control working temperature | -10°C to 50°C |
| Recommended working environment | 0°C to 40°C |
| Rotor diameter | 914.4 mm |
| Folded dimensions | 1,000 × 880 × 700 mm |
| Expanded dimensions | 2,000 × 1,900 × 700 mm |
| Propeller | Foldable 36 × 19 inch diameter × pitch |
| Motor stator and KV | 96 × 20 mm; 100 r/min/V |
| Rated power per listed power system | 1,650 W |
| Maximum working voltage | 60.9 V |
| Maximum power consumption | 9,200 W |
| Take-off weight without load | 37 kg |
| Fully loaded take-off weight | 67 kg |
| No-load hover time | 18 minutes |
| Full-load hover time | 9.5 minutes |
| Maximum pitch angle | 20° |
| Maximum operational flight speed | 10 m/s |
| Maximum tolerable wind | Sustained wind level 4; gust level 5, as stated in the supplied specification. Translate to a site limit only through the approved operating-limit process. |
| Maximum take-off altitude | 2,000 m with load not exceeding 15 kg, as stated in the supplied specification. |
| RTK hover accuracy | ±10 cm horizontally and ±10 cm vertically |
| Hover accuracy without network RTK | ±60 cm horizontally and ±60 cm vertically |
| Obstacle-avoidance radar | 77 GHz dynamic-target detection; stated EIRP 29 dBm; 2.5 W; 5 to 32 V; IP66; perceptible distance 2 to 30 m; vertical FOV ±7.5° and horizontal FOV 112°; front and rear avoidance. |
| Radar stated use conditions | Operating height above 1.5 m, flight speed below 6 m/s and ground slope below 10°; stated safety distance 8 m. Radar does not replace VLOS or pilot avoidance. |

## 6.3 Cleaning and Pump System

| Component | Confirmed Specification or Control |
| --- | --- |
| Onboard spraying description | 12 high-pressure nozzles and two 5 L brushless water pumps; stated spray width 8 to 10 m and maximum flow approximately 7 L/min in the product description. |
| Removable onboard pump | For the 30 L removable tank; brushless plunger pump; approximately 5 L/min; 750 psi maximum; 39.6 to 60.9 V DC; 1,000 W. |
| Onboard tank and flowmeter | 30 L removable tank; rated working pressure 0.2 to 0.4 MPa; impeller flowmeter; stated measurement range 0.5 to 10 L/min and error not exceeding ±5%. |
| Spreading description | Stated spreading width 6 to 10 m. This capability is not used unless it forms part of the specifically approved cleaning configuration. |
| Floor pump type | Brushed plunger pump |
| Floor-pump flow | Approximately 10 L/min |
| Maximum floor-pump outlet pressure | The first supplied JTC30 Tech Spec v01 states 4,100 psi maximum. The later 12-page JTC30 Tech Spec states 3,000 psi maximum. Verify the installed pump nameplate before use. |
| Floor-pump supply | 220 V AC, 4,000 W |
| Engine oil | Four-stroke gasoline engine oil |
| Maintenance interval | 100 hours; first maintenance at 50 hours |
| Separate ground-support pump | The later document identifies a 3WZ high-pressure ground-support unit rated at 14 L/min, 1,450 rpm, 4 kW and 220 V, with 4,100 psi maximum stated pressure. |
| Configuration note | The source documents contain different pump and flow entries. The project method shall identify the actual connected pump, tank, hose and nozzle by model/nameplate and verify pressure/flow before use. |
| Operational pressure control | 3,000 psi, 4,100 psi and 750 psi are stated equipment maxima for different entries, not automatic façade or solar cleaning settings. Use only the lowest effective pressure within surface, nozzle, hose, pump, approval and risk-assessment limits. |

## 6.4 Battery Charger and Remote Control

| Component | Confirmed Specification |
| --- | --- |
| Battery model | JTB1428 / JT1428 secondary lithium-ion battery pack; source uses both identifiers. The later airborne-equipment page also depicts a 14S 36,000 mAh LiPo battery, so installed battery model and approval compatibility must be verified. |
| Battery voltage | 53.2 V |
| Nominal capacity | 28,000 mAh |
| Charging voltage | 60.9 V maximum |
| Charging current | Maximum 55 A |
| Discharge-voltage range | 46.2 V to 60.9 V |
| Battery working temperature | 10°C to 60°C for charging and discharge |
| Battery storage temperature | 0°C to 30°C |
| Battery weight and size | 9.5 kg; 180 × 120 × 290 mm |
| Charger input | 100 to 230 V AC, 50/60 Hz, 13.6 A |
| Charger output | 60.9 V, 55 A, rated power 3 kW |
| Charger protections | Overvoltage, overcurrent, short-circuit, startup delay, shutdown delay, battery communication and USB upgrade. |
| Remote controller | H12 controller with Android embedded system, 5.5-inch high-brightness display and 10,000 mAh built-in battery. |
| Controller communication | Maximum stated communication distance 3 km. This is a technical capability and does not authorize operation beyond the approved VLOS/EVLOS range. |
| Controller endurance | Stated overall battery life up to 6 to 8 hours. |
| Controller charging | Use the charging equipment provided by JOYANCE; fully charge before first use. |

## 6.5 Remote Controller Functions

| Control | Function Stated in Supplied Specification |
| --- | --- |
| A button | Short press toggles the FPV light. |
| B button | Press toggles the spray gun. |
| Power | Short press followed by long press to switch on; long press to switch off. |
| Slide switch | Left and middle positions off; right position return. |
| Three-position switch | Top to bottom: AB point, GPS and attitude flight modes. |
| Joysticks | Aircraft flight control. |
| Antennas | Aircraft command and image-transmission signals. |
| Ports and interfaces | HDMI, USB, Type-C, TF card slot and speaker. |

## 6.6 Propeller Direction and Component Identification

Arms 1, 3 and 5 use counterclockwise rotating propellers. Arms 2, 4 and 6 use clockwise rotating propellers. Fuselage and arm labels identify the arm orientation numbers. The parts identified in the supplied technical document are the smart battery, front casing, machine arm, FPV camera, ultra-high-pressure nozzle, propeller, LED lights, motor and high-pressure water gun.

## 6.7 Midwest Van Mounted Cleaning and Pure Water System

The Midwest system is the company ground-support platform for producing, storing and supplying purified water to the drone operation. It is a van-mounted skid and shall be secured, inspected and operated as an integrated system. The equipment description below is taken from the supplied JTC30 Tech Spec document.

| Subsystem | Confirmed Equipment Details |
| --- | --- |
| Skid and frame | Premium aluminium van skid, TIG welded and manufactured in-house; 48 inches wide by 96 inches long; approximately 900 lb without liquids; oversized fork pockets; intended for high-roof commercial vans. |
| Soft-wash pump | Comet P40 with upgraded 2025 diaphragm; stated output 11.5 US gal/min at 300 psi. |
| Soft-wash engine | Electric-start Honda GX200, gear-driven to pump; internal fuel tank stated as 3.1 L; stated fuel consumption 1.7 L/hour using RON95. |
| Pump mounting | Corrosion-resistant stainless-steel platform with vibration isolators. |
| Remote controls | RigLink remote engine on/off and Remote Rinse Module for switching between chemical and water supply. |
| Proportioner | ClearView Hydro-Mixer XL for proportioning water, chemical and soap without batch mixing. |
| Raw-water tank | One 100 US gallon tank, approximately 379 L nominal. |
| DI-water tank | One 100 US gallon deionized-water tank, approximately 379 L nominal. |
| Chemical tank | One 55 US gallon tank, approximately 208 L nominal. |
| Soap tanks | Two 7 US gallon tanks, approximately 26.5 L each, with soap selector. |
| Purification | Sediment pre-filter, carbon pre-filter and two high-capacity DI resin tanks. |
| Water selector | Valve for directing purified water to the DI/pure-water tank or bypassing filtration to the raw-water tank. |
| Electric reels | Three 12-inch electric reels with aluminium frames and stainless-steel internals. |
| Water-supply hose | One 200 ft × 5/8 inch hose, approximately 61 m. |
| High-pressure hose | One 200 ft × 3/8 inch one-wire hose, approximately 61 m. |
| Soft-wash/drone hose | One 250 ft × 3/8 inch Flexzilla hose, approximately 76 m. |
| Electrical system | Group 27 marine deep-cycle battery in waterproof box; waterproof fused panel with isolation switch; onboard charger. |
| High-pressure accessories | HP gun, insulated wand and JROD with nozzles. |
| Soft-wash accessories | Soft-wash gun, wand and JROD with nozzles. |
| Tank hardware | Raw-water float shutoff and dump valve. |

## 6.8 Midwest System Capacity and Vehicle Load Control

The skid weight is stated without liquid. Filled tanks add substantial mass: water alone is approximately 1 kg per litre, before chemical, fuel, hose and accessories. The operations manager shall calculate the planned transported mass and confirm the actual vehicle payload, axle loads, tyre ratings, load distribution and securing arrangement remain within the vehicle manufacturer and legal limits. Tanks shall not be filled merely to maximum capacity when doing so would exceed a limit or impair stability.

| Load Item | Planning Quantity | Estimated Mass | Verified By |
| --- | --- | --- | --- |
| Skid without liquids | Approximately 900 lb / 408 kg | ________ | ________ |
| Raw water | ______ L | ______ kg | ________ |
| DI/pure water | ______ L | ______ kg | ________ |
| Chemical and soap | ______ L | ______ kg | ________ |
| Fuel, hoses, equipment and crew | As loaded | ______ kg | ________ |
| Total transported payload |  | ______ kg | ________ |
| Vehicle/axle limit |  | ______ kg | ________ |

## 6.9 Equipment Register

| Category | Required Items |
| --- | --- |
| Aircraft system | Airframe, propellers, motors, controller, telemetry, GNSS/navigation, approved firmware and lights. |
| Cleaning delivery | Nozzle, hose, couplings, swivel, strain relief, pump, pressure/flow control, filter, shut-off and water tank/source. |
| Power | Identified flight batteries, controller batteries, charger, generator or supply, leads, protection and fire-safe charging area. |
| Safety | PPE, cones, barriers, warning signs, radios, first aid, eye wash, spill kit and suitable fire equipment. |
| Monitoring | Anemometer, weather information, timer, maps, battery data and maintenance/flight logs. |
| Night | Serviceable aircraft orientation/anti-collision lighting, shielded work lights, illuminated landing area, spare lamps and reflective PPE. |
| Recovery | Approved retrieval equipment and site-specific rescue arrangement; no improvised climbing or hazardous recovery. |

## 6.10 Operating Limits Register

| Parameter | Manufacturer Limit | Company Limit | Project Limit |
| --- | --- | --- | --- |
| Maximum wind | Sustained level 4; gust level 5 | ________ | ________ |
| Maximum height AGL | Approval controlled | ________ | ________ |
| Horizontal range | 3 km link capability; approval/VLOS controls | ________ | ________ |
| Maximum take-off weight | 67 kg fully loaded | ________ | ________ |
| Pressure and flow | Verify installed configuration: onboard 750 psi max/about 5 L/min; floor pump source conflict 3,000 or 4,100 psi/about 10 L/min; separate 3WZ GSE 4,100 psi/14 L/min | ________ | ________ |
| Battery recovery threshold | Not stated in supplied specification | ________ | ________ |
| Temperature range | Recommended environment 0°C to 40°C | ________ | ________ |
| Visibility or cloud restriction | Approval controlled | ________ | ________ |

The lowest or most restrictive applicable limit controls. Values shall not be guessed; they must be transcribed from the current manufacturer documentation, approval and project risk assessment.

# 7 Operational Control and Configuration Management

## 7.1 Controlled Configuration

The technical manager shall maintain a configuration record for aircraft serial number, motors, propellers, controller, firmware, payload, lights, hose, nozzle and approved settings. The PIC verifies the configuration before flight.

## 7.2 Change Control

Record the proposed change and reason.

Assess mass, centre of gravity, performance, command link, lighting, hose forces, electrical and approval impact.

Obtain manufacturer, technical and regulatory acceptance where required.

Conduct a controlled daylight ground test and flight test at a suitable site.

Update manual, training, risk assessment and configuration register before operational release.

## 7.3 Document Hierarchy

| Priority | Controlling Requirement |
| --- | --- |
| 1 | Applicable law and issued approval conditions |
| 2 | ATC or authority instruction during the operation |
| 3 | Manufacturer limitations and safety instructions |
| 4 | Site-specific risk assessment, method statement and emergency plan |
| 5 | This operations manual and company procedures |
| 6 | Customer preferences where consistent with all higher requirements |

# 8 Customer Order and Project Lifecycle

Figure 1 Drone cleaning service workflow

## 8.1 Process

| Stage | Output or Release Evidence |
| --- | --- |
| Customer enquiry | Site, scope, contact, preferred dates and customer expectations recorded. |
| Initial review | Preliminary feasibility and information gaps identified. |
| Site survey | Hazards, areas, dimensions, utilities, public exposure and airspace recorded. |
| Quotation | Scope, method, price, exclusions, schedule and terms issued. |
| Order acceptance | PO, LOA or signed quotation received. |
| Operational planning | ConOps, risk assessment, method statement, map, crew and emergency plan completed. |
| Approvals | All written approvals and site permits verified. |
| Mobilization | Serviceable equipment and competent crew dispatched. |
| Execution | Pre-flight release, cleaning, monitoring and logs completed. |
| Inspection | Customer/site representative verifies agreed work or rectification is completed. |
| Close-out | Equipment removed, site made safe, records completed. |
| Final report | Customer receives documented evidence and recommendations. |

# 9 Site Survey and Feasibility Assessment

## 9.1 Survey Timing

A daylight survey shall normally be completed before the first operation. Night projects also require an after-dark survey to assess lighting, reflections, traffic, occupancy, noise and visibility changes.

## 9.2 Survey Items

| Area | Survey Requirement |
| --- | --- |
| Façade | Material, condition, defects, sealants, windows, vents, projections, recesses and fragile components. |
| Airspace | Coordinates, nearby aerodromes/heliports, restrictions, likely manned traffic and communication requirements. |
| Ground | Public routes, roads, parking, entrances, emergency access, exclusion-zone capacity and alternative landing areas. |
| Obstacles | Wires, antennas, cranes, trees, signage, ledges and roof equipment. |
| Utilities | Water quality/source, drainage, electrical systems, generator location and isolation capability. |
| Hose route | Length, weight, snag points, sharp edges, vehicle crossings, restraint and ground-handler positions. |
| Environment | Wind acceleration, turbulence, heat, rain, lightning, glare, wildlife, plants and sensitive receptors. |
| Night | Work-light positions, shadows, reflections, illuminated landing zone, neighbouring occupants and security. |
| Emergency | Access for rescue, fire and medical services; nearest safe assembly point and hospital. |
| Customer | Acceptance criteria, operating hours, notifications, photographs and handover representative. |

## 9.3 Feasibility Decision

The project manager records Accepted, Accepted with Controls, or Not Feasible. A not-feasible activity may be reconsidered only after the hazard or approval gap is demonstrably resolved.

# 10 Concept of Operations and Flight Planning

## 10.1 Required ConOps Content

Purpose, aircraft, payload and cleaning method.

Coordinates, activity area, geofence, contingency area, ground-risk buffer and emergency landing sites.

Heights, stand-off distances, flight direction, cleaning passes and hose route.

Crew roles, communications, public control and customer interfaces.

Operating window, weather limits, daylight or night status and fatigue controls.

Normal, contingency and emergency procedures.

Approval-specific conditions, reporting and record retention.

## 10.2 Flight Map

The flight map shall use the format required by the approval process and show the activity, geofencing, risk and outer boundary zones where applicable. The PIC shall have an accessible field copy and verify it against the actual site before launch.

## 10.3 Work Sequencing

Plan short cleaning sectors that allow frequent landing, inspection and battery change. Work from a position that keeps the aircraft, nozzle, hose and hazards visible. Avoid routing the hose across public access or around corners without dedicated control.

## 10.4 Communications

| Standard Call | Meaning and Response |
| --- | --- |
| STOP STOP STOP | Immediate stop. Pump off; PIC stabilizes and lands or applies contingency. |
| TRAFFIC | Potential aircraft conflict. Pause task, yield and follow approved response. |
| PERSON IN ZONE | Ground intrusion. Pump off; hold safely or land. |
| HOSE TENSION | Pump off; reduce load and reposition or land. |
| LOW BATTERY | End cleaning pass and recover within approved reserve. |
| LIGHTS | Lighting degraded or failed; recover and land. |
| LAND NOW | Immediate controlled landing at primary or emergency area. |
| ALL CLEAR | PIC issues only after each assigned role confirms readiness. |

# 11 Risk Assessment and Work Authorization

## 11.1 Risk Process

- Identify hazards for preparation, launch, transit, cleaning, landing and close-out.

- Identify credible causes and consequences for people, aircraft, property and environment.

- Assess initial likelihood and severity using the company matrix.

- Apply elimination, engineering, administrative and PPE controls.

- Assess residual risk and assign action owners.

- Obtain approval at the authority level required by the risk rating.

- Brief the final assessment and reassess when conditions change.

## 11.2 Minimum Hazards

| Hazard | Core Controls |
| --- | --- |
| Aircraft impact or fly-away | Approved boundary, serviceable system, VLOS, trained crew, controlled area and lost-link procedure. |
| Manned aircraft conflict | Airspace review, continuous scan, approval conditions and immediate yield/landing action. |
| Hose snag, whip or overload | Planned route, strain relief, pressure isolation, ground handlers and conservative manoeuvres. |
| Public intrusion | Barriers, marshals, alternate routes and immediate stop. |
| Wind or turbulence | Forecast and on-site measurement; corner/roof-edge margin; stop limits. |
| Electrical contact | Identify, isolate or maintain approved clearance; no conductive water exposure to live systems. |
| Water or chemical damage | Material test, controlled pressure, approved detergent, runoff plan and customer authorization. |
| Battery fire | Inspection, protected charging, quarantine, fire response and emergency access. |
| Night disorientation | Training, lighting, VLOS, work-light control, observer cues and prompt landing. |
| Fatigue and communication failure | Roster, fitness declaration, dedicated roles and standard calls. |
| Falling façade material | Condition survey, exclusion zone and stop on movement or damage. |
| Vehicle or road exposure | No uncontrolled overflight; traffic management approved by relevant authority. |

## 11.3 Dynamic Risk Review

The PIC repeats a dynamic review before each flight and after any weather change, interruption, public intrusion, equipment warning, battery change, crew change or scope change.

# 12 Weather and Environmental Conditions

## 12.1 Weather Sources and On Site Checks

The operations manager reviews forecast and current information for the entire setup, flight and recovery period. The PIC measures conditions at representative ground locations and accounts for higher wind, turbulence and downdraft around roof edges and corners.

## 12.2 No Go Conditions

Lightning risk, thunderstorm, electrical storm or unsafe convective weather.

Wind or gusts at or above the lowest applicable limit.

Heavy rain, fog, smoke, glare or darkness that prevents reliable visual control.

Conditions that may cause detergent drift or runoff outside the controlled area.

Temperature outside battery, aircraft, pump or chemical limits.

Unstable or rapidly changing weather without adequate safe recovery margin.

## 12.3 Heat and Hydration

Provide shade, drinking water and rest. Monitor pilot, observer and ground crew for heat stress. Water-resistant clothing and PPE must not create excessive heat burden.

## 12.4 Environmental Protection

Prevent uncontrolled discharge to drains, soil, vegetation and waterways. Stop immediately if runoff containment fails or chemical drift affects people, plants, vehicles or neighbouring property.

# 13 Ground Safety and Exclusion Zones

## 13.1 Controlled Area

The ground-risk area shall contain the predicted aircraft, hose, water spray, falling-object and emergency-landing hazards. Its size is determined by approval, risk assessment, height, aircraft energy, wind, hose behavior and site layout—not by convenience.

## 13.2 Setup

☐  Install barriers, cones and warning signs before aircraft setup.

☐  Place trained marshals at every uncontrolled access point.

☐  Maintain fire, medical and emergency-vehicle access.

☐  Protect hoses and cables from vehicles, sharp edges and trip exposure.

☐  Use an illuminated landing pad for night work.

☐  Stop the operation whenever the controlled area cannot be maintained.

## 13.3 PPE

| Role | Minimum PPE Subject to Site Assessment |
| --- | --- |
| All field crew | Safety footwear, high-visibility clothing, suitable head protection and eye protection. |
| Pump/chemical operator | Chemical-resistant gloves, eye/face protection and apron/clothing stated by SDS. |
| Night crew | Reflective high-visibility clothing and personal torch without creating pilot glare. |
| Electrical or specialist task | Only task-specific PPE selected by a competent person; PPE does not replace isolation. |
| Hearing exposure | Hearing protection where assessed noise exceeds acceptable exposure. |

# 14 Mobilization and Site Setup

## 14.1 Mobilization Release

☐  Customer order and final scope confirmed.

☐  Approvals and permits verified.

☐  Competent crew and duty roster confirmed.

☐  Aircraft, payload, batteries and ground equipment released serviceable.

☐  Weather window acceptable.

☐  Water, power, access, parking and loading arrangements confirmed.

☐  Emergency contacts and nearest medical facility recorded.

## 14.2 Site Setup Sequence

- Sign in and conduct site induction.

- Walk the site against the approved map and identify any change.

- Establish exclusion zone, signs and marshal positions.

- Set up water, pump, hose route, strain relief, spill control and emergency isolation.

- Set up pilot, observer and landing positions with unobstructed view.

- Position emergency equipment and communication devices.

- Complete ground test and crew briefing.

- Record PIC go or no-go decision.

## 14.3 Midwest Skid and Van Setup

☐  Park on stable, level ground within the approved service area and apply parking brake/wheel restraints as assessed.

☐  Verify the skid and all tanks, reels, batteries, accessories and loose items remain positively secured to the vehicle.

☐  Inspect tank level, tank condition, lids, valves, float shutoff, dump valve and signs of leakage.

☐  Vent engine exhaust safely away from the van interior, crew, public, air intakes and combustible material. Never run the petrol engine in an enclosed or poorly ventilated space.

☐  Establish fuel/fire controls; inspect the Honda engine and Comet P40 before starting.

☐  Deploy only the required hose length, protect crossing points and maintain safe reel tension.

☐  Identify and label the raw-water, DI-water, chemical and soap circuits.

☐  Test electrical isolation, waterproof fuse panel, remote engine control and rinse selector.

☐  Confirm the selected outlet and hose route lead to the intended drone supply connection before pressurizing.

# 15 Pre Flight Inspection and Briefing

## 15.1 Aircraft Check

☐  Airframe, arms, motors, propellers, guards and fasteners free from damage.

☐  Payload, nozzle and mounting secure; mass and centre of gravity within approved configuration.

☐  Navigation, status and anti-collision lights serviceable.

☐  Controller, antennas, telemetry, firmware/configuration and control inputs verified.

☐  GNSS/navigation status, geofence and contingency settings verified.

☐  Flight and controller batteries identified, charged, undamaged and within temperature limits.

## 15.2 Cleaning System Check

☐  Water source and quality confirmed.

☐  Filter, hose, coupling, swivel, strain relief, nozzle and shut-off inspected.

☐  Pump pressure and flow tested at ground level.

☐  Leaks, whip, movement and emergency isolation tested.

☐  Detergent identity, dilution, SDS and material compatibility confirmed.

## 15.3 Briefing

The PIC briefs approval limits, weather, map, work sector, roles, radio calls, exclusion zone, hose handling, battery plan, emergency landing areas, lost-link action, fire/electrical response and stop-work authority. Each crew member confirms understanding.

# 16 Standard Daytime Flight Operations

## 16.1 Launch and Functional Check

- PIC confirms area sterile and announces arming.

- Launch with pump off and establish a low hover.

- Verify control response, stability, navigation, lighting, telemetry and hose behavior.

- Abort and land if any indication is abnormal.

- Proceed to work position at low speed only after PIC announces functional check complete.

## 16.2 Cleaning Flight

- Establish stable position and stand-off distance.

- PIC commands pump on; pump operator confirms flow and pressure.

- Use short, deliberate passes with predictable movement.

- Observer reports obstacles, air traffic, people and hose clearances.

- Stop flow before major repositioning, corners, recesses or obstacle transitions.

- Monitor battery, wind, link, vibration, nozzle behavior and surface response continuously.

- Recover at the approved threshold with reserve for contingency.

## 16.3 Battery Change

Land, disarm, isolate and depressurize the pump before approach. Inspect the aircraft, nozzle, hose and coupling; record battery ID and flight time; release the next battery only after condition and temperature checks.

# 17 Façade and Glass Cleaning Operations

## 17.1 General Method

- Confirm material compatibility and identify existing cracks, loose seals, open vents and damaged panels.

- Conduct a small test area where pressure, detergent or finish response is uncertain.

- Use the lowest effective pressure and a controlled stand-off distance.

- Where detergent is required, apply the approved dilution and observe dwell time without allowing it to dry.

- Rinse with suitable water and verify runoff control.

- Inspect for streaks, residue, damage and missed areas before moving sectors.

## 17.2 Glass

Pure or low-TDS water may be used where compatible with the cleaning objective and site method. Avoid excessive pressure on edges, seals, openable panels or damaged glazing. Manage reflections that may mislead visual orientation or conceal obstacles.

## 17.3 Painted Walls and Cladding

Verify coating condition, age and compatibility. Do not direct high pressure into joints, vents, electrical penetrations, loose paint or behind cladding. Stop on discoloration, lifting, water ingress or material movement.

## 17.4 Grease and Localized Stains

Do not increase pressure or chemical strength without an approved test and product instruction. Specialized degreasing requires separate compatibility, runoff and exposure controls. Hot water is used only when the equipment, surface, chemical and method specifically allow it.

# 18 Solar Panel Cleaning Operations

## 18.1 Planning

Confirm panel manufacturer requirements, array owner authorization, electrical status, roof/site controls and acceptable water quality. Drone operation does not remove the electrical hazard from energized photovoltaic equipment.

## 18.2 Method

Prefer suitable pure or low-mineral water when consistent with panel manufacturer instructions.

Avoid abrasive chemicals, hard brushes, thermal shock and excessive pressure.

Do not clean damaged panels, exposed conductors, loose connectors or unidentified electrical faults.

Plan spray direction to prevent water entry into junction boxes, connectors and building openings.

Stop if arcing, smoke, odor, abnormal heat, damaged cable or water ingress is observed.

Inspect and report visible defects without representing the cleaning inspection as an electrical certification.

## 18.3 Temperature

Avoid cleaning very hot panels with substantially cooler water where thermal shock is credible. Select an approved time window and verify surface and ambient conditions.

# 19 Water Detergent and Runoff Controls

## 19.1 Product Approval

Only products approved by the company and accepted for the surface may be used. The current SDS and technical data shall be available at site. Product selection must consider glass, paint, metal, rubber, silicone, plants, drainage, workers and the public.

## 19.2 Mixing and Application

☐  Identify and label containers.

☐  Use measured dilution and never mix incompatible products.

☐  Use PPE specified by the SDS and risk assessment.

☐  Prevent overspray and drift; stop when wind defeats control.

☐  Do not allow detergent to dry on the surface unless product instructions permit it.

☐  Keep eyewash, clean water and spill materials immediately available.

## 19.3 Runoff

Agree the drainage and containment plan with the customer. Protect sensitive drains and receptors. Collect or control contaminated runoff where required. Do not claim environmental compliance solely because a product is marketed as green or biodegradable.

## 19.4 Midwest Pure Water Production

The Midwest three-stage system produces the purified-water supply used for the drone operation. The process uses sediment filtration, carbon filtration and two DI-resin tanks before water is directed to the DI/pure-water storage tank. The raw-water bypass shall not be selected when the method requires pure water.

- Confirm the raw-water source is suitable for the filtration system and record any source restriction.

- Inspect sediment and carbon pre-filters, DI vessels, connections, selector valve and tank cleanliness.

- Set the selector valve to the verified pure-water/DI tank route.

- Flush the system according to supplier instructions before collecting operational water, particularly after storage or filter/resin replacement.

- Measure outlet-water TDS with a calibrated or function-checked meter after flushing.

- Continue filling only when the reading meets the approved project limit. Record the initial, periodic and final readings.

- Isolate the chemical and soap circuits from the pure-water route and verify no cross-contamination.

- Close and identify the DI tank; prevent raw-water top-up into the pure-water tank.

- Before drone connection, flush the selected hose/nozzle until the outlet reading again meets the approved limit.

- Stop production when TDS rises above the project limit, flow changes abnormally, a filter differential/condition indicates blockage, or contamination is suspected.

## 19.5 Pure Water Quality Control

| Control Point | Required Record or Acceptance |
| --- | --- |
| Raw-water source | Source and date recorded; visible contamination absent. |
| TDS meter | Meter ID and function/calibration check recorded. |
| Post-DI outlet | ______ ppm maximum project limit; actual ______ ppm. |
| DI tank | Clean, correctly labelled and isolated from chemical/soap circuits. |
| Drone supply hose outlet | Actual ______ ppm after hose flush. |
| During operation | Reading frequency: ______; results: __________________. |
| End of operation | Final reading ______ ppm; abnormal change investigated. |
| Corrective action | Replace/regenerate DI resin, replace filter, flush or quarantine as required. |

A universal TDS acceptance value is not stated in the supplied equipment document. The accountable manager shall set the project limit using customer requirements, surface/manufacturer guidance and the required finish. A low TDS reading indicates dissolved-ion content; it does not by itself prove microbiological or chemical purity.

## 19.6 Switching Between Pure Water and Chemicals

Obtain PIC and cleaning supervisor authorization before changing supply mode.

Stop the drone cleaning pass and depressurize the selected circuit.

Use the Remote Rinse Module and Hydro-Mixer controls only in accordance with the supplier instructions.

Verify valve alignment and prevent backflow into the DI tank or filtration train.

Flush the hose to the defined changeover standard and verify TDS when returning to pure water.

Label and record the chemical, soap, proportion setting and quantity used.

Never connect an unidentified hose or open a valve based only on color; trace and confirm the full route.

## 19.7 Midwest System Shutdown

☐  Stop pump/engine and isolate electrical and fuel sources.

☐  Release pressure before disconnecting any hose or fitting.

☐  Return valves and Remote Rinse Module to the approved shutdown position.

☐  Drain, flush or preserve circuits according to supplier instructions and the chemicals used.

☐  Record tank levels, TDS, filter/resin condition, leaks, defects and maintenance due.

☐  Rewind hoses evenly and secure reels, guns, wands, JRODs and accessories.

☐  Secure tanks, lids, dump valve and all loose equipment before vehicle movement.

# 20 Night Operations

## 20.1 Authorization

Night cleaning is prohibited unless the written CAAM approval and all supporting permissions expressly authorize the operation, aircraft, cleaning payload, site and night window. The PIC shall verify official local sunset and sunrise times and use the approval’s definition and conditions.

## 20.2 Additional Crew and Equipment

Dedicated PIC and visual observer with current night familiarization.

Serviceable orientation/navigation and anti-collision lighting visible through the operating volume.

Shielded work lighting that illuminates aircraft, façade, obstacles and hose without dazzling crew.

Illuminated landing and emergency landing areas.

Reflective PPE, spare lamps, spare batteries and reliable radios.

Sufficient marshals for after-hours entrances, vehicles and security changes.

## 20.3 Night Site Assessment

| Hazard | Night Control |
| --- | --- |
| Loss of orientation | Distinct serviceable aircraft lighting, conservative range, observer callouts and immediate landing on uncertainty. |
| Glass reflections | Position lights and crew to reduce false images; use stable reference points and lower speed. |
| Glare and shadows | Shield and angle lamps; check from PIC/observer positions before launch. |
| Hidden obstacles | Complete daylight and darkness surveys; illuminate critical wires, corners and projections. |
| Fatigue | Planned roster, fitness declaration, breaks and conservative continuous-duty period. |
| Public/security changes | Confirm after-hours access, deliveries, vehicles and occupant movements. |
| Noise | Approved work hours and notification to affected occupants. |
| Lighting failure | Pump off; maintain observer callouts; land without delay under backup lighting. |

## 20.4 Night Launch and Flight

- Complete all daytime checks plus darkness lighting test.

- Launch with pump off and hover low to confirm orientation and visibility.

- Keep the aircraft within the distance where attitude and hazards remain clearly discernible.

- Use slower inputs, shorter sectors and increased stand-off where reflections or shadows reduce depth perception.

- Reassess pilot and observer alertness at each battery change.

- Abort on any loss of lighting, orientation, visual reference or reliable communication.

## 20.5 Night Stop Criteria

Land immediately for loss of VLOS, orientation, required lighting, observer communication, controlled area, reliable position reference, unexpected traffic, unsafe weather, crew alertness or any approval condition.

# 21 Abnormal and Emergency Procedures

| Event | Immediate Actions |
| --- | --- |
| Loss of visual orientation | Pump off. Stabilize or reduce movement. Use observer cues. If not immediately recovered, execute approved landing/contingency. |
| Lost C2 link | Stop pump if independently possible. Apply approved lost-link profile. Clear predicted route and landing area. |
| GNSS or navigation anomaly | Discontinue cleaning; stabilize in an approved mode and land. |
| Fly-away or containment loss | Stop pump, warn relevant contacts, track last position without endangering crew, and notify authorities as required. |
| Hose snag or excessive tension | Pump off and depressurize; avoid pulling against obstacle; land if safe; retrieve only after disarm and hazard isolation. |
| Pump or coupling failure | Pump off and isolate; land; control leak and inspect before release. |
| Crash or hard landing | Emergency stop and pump isolation; keep area clear; first aid; emergency services; preserve evidence; quarantine system. |
| Battery fire | Do not handle or recharge; apply site battery-fire response; avoid fumes; evacuate and call Fire and Rescue where necessary. |
| Chemical exposure | Stop source; follow SDS; flush eyes/skin as directed; obtain medical support and contain spill. |
| Electrical contact or arcing | Do not touch aircraft, hose, water or casualty until supply is verified isolated by an authorized person; call 999. |
| Manned aircraft conflict | Give way; descend or land promptly as approved while maintaining ground safety. |
| Injury or property damage | Stop, make safe, provide first aid, call 999 where warranted, notify management/client/insurer and preserve records. |
| Sudden weather | Pump off; recover by the safest approved route; secure equipment and account for crew. |

## 21.1 Emergency Priorities

Protect life first, prevent escalation, communicate clearly, isolate energy and pressure, preserve the scene and evidence, then report through the required channels. Personnel shall not enter a hazardous area or climb for recovery without competent authorization and proper controls.

## 21.2 Emergency Contacts

| Contact | Number or Details |
| --- | --- |
| Emergency Services | 999 |
| Building Control | ________________ |
| Site Security | ________________ |
| Project Manager | ________________ |
| Accountable Manager | ________________ |
| CAAM or ATC contact in approval | ________________ |
| Nearest Hospital | ________________ |
| Insurer | ________________ |

# 22 Incident Near Miss and Damage Reporting

## 22.1 Immediate Reporting

All injuries, fly-aways, loss of containment, airspace conflicts, crashes, hard landings, fires, electrical events, chemical releases, property damage, near misses and safety-critical defects shall be reported immediately to the PIC and operations manager.

## 22.2 Evidence

Flight and controller logs

Telemetry and warning messages

Aircraft, battery and configuration identification

Photographs and video authorized for investigation

Weather observations and approval documents

Witness names and statements

Damaged components and maintenance status

Communications and timeline

## 22.3 External Notification

The accountable manager determines and completes notifications to CAAM, air traffic services, police, emergency services, client, insurer or other authorities within the applicable timeframe. Crew shall not delete, alter or overwrite records.

## 22.4 Return to Service

An aircraft involved in a crash, hard landing, overload, water ingress, electrical event, fly-away or abnormal vibration remains quarantined until inspected and formally released by authorized technical personnel.

# 23 Post Flight Demobilization and Customer Reporting

## 23.1 Post Flight

☐  Land, disarm, isolate pump and release pressure before approach.

☐  Inspect airframe, motors, propellers, lights, nozzle, hose, couplings and batteries.

☐  Record flight time, battery cycles, warnings, weather, interruptions and defects.

☐  Clean and dry equipment; quarantine any suspect item.

☐  Account for water and detergent and complete runoff/waste controls.

## 23.2 Inspection and Rectification

Inspect the completed surface from agreed viewpoints. Compare with the scope and acceptance criteria. Rectify only within the approved method and operating window. Record inaccessible, unsafe or pre-existing defects separately.

## 23.3 Demobilization

Remove aircraft, water system, hoses, barriers and waste only after the site is safe. Restore access routes and obtain site representative acknowledgment where required.

## 23.4 Final Service Report

| Report Section | Minimum Content |
| --- | --- |
| Project details | Customer, site, order reference, dates, contacts and approved scope. |
| Operational details | Aircraft, crew, approval reference, flight times, weather and method. |
| Work completed | Areas, approximate dimensions, passes, water and detergent used. |
| Evidence | Before, during and after photographs with appropriate captions. |
| Observations | Stains, defects, inaccessible areas, damage, leaks or safety concerns observed. |
| Variations | Approved changes, interruptions, rectification and exclusions. |
| Recommendations | Further treatment, repair, specialist inspection or future frequency where justified. |
| Acceptance | Customer/site representative name, comments, signature and date. |

# 24 Maintenance Inspection and Defect Control

## 24.1 Maintenance Principles

Maintenance follows the current manufacturer instructions and approved company program. Only authorized personnel may perform or certify assigned tasks. Service intervals are based on calendar time, flight time, cycles, condition and event triggers.

| Frequency or Trigger | Minimum Action |
| --- | --- |
| Before each flight | Airframe, propellers, motors, fasteners, payload, lights, battery, controller, hose and nozzle. |
| After each shift | Clean and dry; inspect water ingress, corrosion, wear, coupling movement and battery condition. |
| Weekly or manufacturer interval | Detailed aircraft, motor, pump, cable, light and control-system inspection. |
| Hard landing, collision, snag or overload | Quarantine and event inspection before documented return to service. |
| Water ingress or chemical contamination | Isolate, dry/decontaminate as approved and perform technical assessment. |
| Firmware or configuration change | Controlled daylight test and approval-impact review before operational use. |
| Long storage | Preservation, battery storage condition and return-to-service inspection. |

## 24.2 Defect Control

Defects are recorded with date, system ID, description, reporter and operational effect. Items are tagged Serviceable, Restricted, Under Maintenance or Quarantined. No person may erase a defect without recording corrective action and authorized release.

## 24.3 Midwest Ground System Maintenance

| Component | Inspection and Maintenance Control |
| --- | --- |
| Comet P40 and Honda GX200 | Follow supplier and engine manuals; inspect oil, fuel, leakage, vibration, mountings, start/stop and control function. |
| Sediment/carbon filters | Inspect pressure/flow and visible condition; replace at supplier interval or earlier when contaminated or restricted. |
| DI resin tanks | Trend outlet TDS; replace or regenerate resin when the acceptance limit cannot be maintained after proper flushing. |
| Raw/DI/chemical/soap tanks | Inspect cleanliness, cracks, lids, labels, fittings, float shutoff, dump valve and internal contamination. |
| Proportioner and rinse module | Function-test selection, proportion control, remote switching, backflow prevention and isolation. |
| Electric reels and hoses | Inspect frames, stainless internals, electrical function, hose abrasion, kinks, bulges, fittings and reel restraint. |
| Marine battery and fused panel | Inspect enclosure, terminals, charge condition, fuse rating, isolation switch and water ingress. |
| Skid/frame and vehicle securing | Inspect welds, corrosion, fork pockets, mounting points, fasteners and load restraints. |
| TDS meter | Maintain, clean and verify/calibrate according to meter instructions; record meter ID and status. |
| System preservation | Drain, flush, protect from freezing/heat and prevent biological growth or chemical crystallization during storage. |

# 25 Battery Charging Transport and Storage

## 25.1 Inspection and Identification

Each battery has a unique ID and cycle/condition record. Remove from service any battery showing swelling, impact, leakage, unusual heat, damaged connector, imbalance, warning or performance deterioration.

## 25.2 Charging

☐  Use the manufacturer-approved charger and settings.

☐  Charge in a ventilated, supervised, fire-aware area away from combustibles and public access.

☐  Allow batteries to reach permitted temperature before charging.

☐  Do not charge damaged, wet, swollen or suspect batteries.

☐  Keep suitable fire response and emergency access available.

☐  Record abnormal charge behavior and quarantine the battery.

## 25.3 Transport and Storage

Protect terminals from short circuit, secure batteries against movement and impact, and comply with applicable transport requirements. Store at the manufacturer-recommended state of charge, temperature and separation. Do not leave batteries in hot vehicles.

## 25.4 Battery Emergency

On smoke, hissing, rapid heating or fire, raise alarm, isolate people, avoid toxic fumes and apply the site emergency plan. Re-ignition is possible; treat affected batteries as hazardous and do not return them to normal storage.

# 26 Records Audit and Continuous Improvement

## 26.1 Required Records

Approvals, permits and client authorization

ConOps, maps, risk assessment and method statement

Crew competency, fitness, briefing and duty records

Aircraft, payload, battery, configuration and maintenance records

Weather checks, flight logs and cleaning records

SDS, water quality and runoff records where applicable

Incident, near-miss, defect and corrective-action reports

Customer inspection, final report and acceptance

## 26.2 Retention

Records are retained for the longest period required by law, approval, insurance, contract and company policy. The accountable manager shall approve the retention schedule and access controls.

## 26.3 Audit

The company shall audit operational files, aircraft records, training, approval compliance and field practice at planned intervals. Findings receive an owner, due date and verified closure.

## 26.4 Review Triggers

Review this manual after an incident, significant near miss, regulatory change, new aircraft or payload, new cleaning chemical, night-operation change, recurring defect, audit finding or material customer complaint. Lessons learned shall be communicated before the next affected operation.

# Appendix A Project Readiness Checklist

☐  Customer order, scope and acceptance criteria confirmed.

☐  Daylight site survey completed; night survey completed when applicable.

☐  CAAM and supporting approvals cover aircraft, payload, site, date, time and crew.

☐  ConOps, risk assessment, method statement and emergency plan approved.

☐  Insurance and customer authorization verified.

☐  Crew competency, fitness and roles confirmed.

☐  Aircraft and support equipment serviceable.

☐  Weather window and backup date acceptable.

☐  Water, detergent, drainage, electricity and access confirmed.

☐  Emergency contacts and reporting requirements recorded.

☐  Operations Manager release:  ☐ RELEASED  ☐ NOT RELEASED   Name: __________ Date: ______

# Appendix B Site Survey Form

| Field | Survey Record |
| --- | --- |
| Customer and site |  |
| Coordinates |  |
| Building/surface and approximate area |  |
| Façade condition and defects |  |
| Airspace and nearby aerodrome/heliport |  |
| Maximum planned height |  |
| Obstacles and stand-off distances |  |
| Public/vehicle routes and entrances |  |
| Exclusion-zone dimensions |  |
| Launch/landing and emergency landing areas |  |
| Water source and quality |  |
| Drainage/runoff route |  |
| Electrical hazards and isolation |  |
| Hose route and restraint points |  |
| Weather/wind effects |  |
| Night lighting/reflection/noise hazards |  |
| Emergency access and nearest hospital |  |
| Customer restrictions and acceptance criteria |  |
| Feasibility | ☐ Accepted ☐ Accepted with controls ☐ Not feasible |
| Surveyor/date/signature |  |

# Appendix C Pre Flight and Go No Go Checklist

☐  Approval and operating map verified.

☐  Weather acceptable and reassessment time set.

☐  Exclusion zone secure and marshals positioned.

☐  Crew fit, briefed and on tested radios.

☐  Airframe, motors, propellers and fasteners inspected.

☐  Payload, nozzle, hose and strain relief secure.

☐  Controller, telemetry, GNSS/navigation and settings verified.

☐  Aircraft and controller batteries acceptable.

☐  Lights tested; full night-lighting check completed when applicable.

☐  Pump, flow, pressure, shut-off and leak test completed.

☐  Detergent, SDS, PPE, spill and eye-wash controls ready.

☐  Emergency landing areas and contacts confirmed.

☐  PIC decision:  ☐ GO  ☐ NO GO   PIC: __________ Time: ______

# Appendix D Night Operation Checklist

☐  Approval expressly permits night cleaning and the planned night window.

☐  Day and darkness site surveys complete.

☐  Aircraft orientation/navigation and anti-collision lights serviceable.

☐  Work lights illuminate aircraft, surface, obstacles and hose without disabling glare.

☐  Landing and emergency landing areas illuminated.

☐  PIC and observer night-current and fit for duty.

☐  Reflective PPE, backup lights and spare batteries available.

☐  Public/security access controlled for after-hours conditions.

☐  VLOS/approved EVLOS and orientation can be maintained throughout the sector.

☐  Noise/occupant notifications complete.

☐  PIC night release:  ☐ GO  ☐ NO GO   PIC: __________ Time: ______

# Appendix E Flight and Cleaning Log

| Field | Entry |
| --- | --- |
| Project/site |  |
| Date and operating window |  |
| Approval reference |  |
| Aircraft serial/configuration |  |
| PIC/Observer/Pump Operator |  |
| Day or night |  |
| Take-off and landing times |  |
| Maximum height/work sector |  |
| Battery IDs and cycles |  |
| Weather and wind |  |
| Water/detergent/dilution |  |
| Pressure and flow |  |
| Warnings/aborts/interruptions |  |
| Defects/quarantine |  |
| Work completed |  |
| PIC signature |  |

# Appendix F Maintenance and Defect Record

| Date | System ID | Flight/Cycles | Inspection or Defect | Action and Release |
| --- | --- | --- | --- | --- |

# Appendix G Incident and Near Miss Form

| Field | Record |
| --- | --- |
| Date, time, site and reporter |  |
| Aircraft, battery and configuration |  |
| Type | ☐ Incident ☐ Near miss ☐ Defect ☐ Complaint |
| Event description and timeline |  |
| Immediate action |  |
| Injury/property/environment impact |  |
| People and authorities notified |  |
| Weather and operating phase |  |
| Evidence retained | ☐ Photos ☐ Flight logs ☐ Telemetry ☐ Witnesses |
| Aircraft status | ☐ Released ☐ Restricted ☐ Quarantined |
| Root cause and contributing factors |  |
| Corrective action, owner and due date |  |
| Management closure |  |

# Appendix H Customer Service Report Template

| Report Field | Details |
| --- | --- |
| Customer/project/site |  |
| Order and approval references |  |
| Operation dates and times |  |
| Crew and aircraft |  |
| Weather |  |
| Scope completed |  |
| Cleaning method and water/chemical |  |
| Flight summary |  |
| Before/during/after photographs | Attach and caption separately |
| Observations and defects |  |
| Variations or inaccessible areas |  |
| Rectification completed |  |
| Recommendations |  |
| Customer comments |  |
| Customer representative/signature/date |  |
| Company representative/signature/date |  |

# Appendix I Site Specific Operating Limits

| Item | Approved Value or Control |
| --- | --- |
| Site coordinates and boundary |  |
| Approval reference and validity |  |
| Aircraft and payload configuration |  |
| Day/night operating window |  |
| Maximum height |  |
| Horizontal/stand-off distance |  |
| Wind and gust limit |  |
| Visibility/weather limits |  |
| Battery recovery threshold |  |
| Pressure and flow |  |
| Water and detergent |  |
| Exclusion-zone dimensions |  |
| Emergency landing areas |  |
| Airspace/ATC communications |  |
| Approval-specific conditions |  |

# Appendix J Midwest Pure Water Operation Log

| Field | Record |
| --- | --- |
| Project, date and operator |  |
| Raw-water source |  |
| Vehicle payload/load check | ☐ Completed Reference: __________ |
| Sediment pre-filter condition |  |
| Carbon pre-filter condition |  |
| DI resin vessels condition |  |
| TDS meter ID/check |  |
| Approved project TDS limit | ______ ppm |
| Initial post-DI TDS | ______ ppm at ______ time |
| DI tank quantity produced | ______ L |
| Chemical/soap isolation verified | ☐ Yes ☐ No |
| Drone-hose outlet after flush | ______ ppm |
| Intermediate readings | Time ______ / ______ ppm Time ______ / ______ ppm |
| Final reading | ______ ppm |
| Filters/resin changed | ☐ No ☐ Yes - details: __________________ |
| Leaks, defects or corrective action |  |
| Shutdown and securing completed | ☐ Yes |
| Operator and supervisor signatures |  |

# Appendix K Reference Register

| Reference | Purpose |
| --- | --- |
| Malaysian Civil Aviation Regulations 2016 | Primary civil aviation regulatory framework. |
| CAAM Unmanned Aircraft System portal | Current approval routes, forms and application guidance. |
| CAD 6011 and applicable subseries | UAS approval, training and risk requirements relevant to the operation. |
| Issued CAAM approval and attachments | Controlling project-specific limitations and conditions. |
| JTC30 manufacturer operation and maintenance manuals | Aircraft, payload, battery, performance and maintenance limits. |
| Site HSE requirements and permits | Customer/building rules and emergency arrangements. |
| Current SDS and technical data sheets | Chemical hazards, PPE, dilution, compatibility and emergency action. |
| Insurance policy and endorsements | Coverage, exclusions, reporting and claims requirements. |

Technical data incorporated in Revision 01 were taken from JTC30 Tech Spec v01, a six-page technical document supplied to SERI N.S SDN. BHD. Midwest equipment data incorporated in Revision 02 were taken from the 12-page JTC30 Tech Spec document supplied to the company. Source documents shall remain in the controlled technical file and be checked against equipment nameplates, the installed configuration and latest supplier instructions before operational release.

Before each controlled revision, the document owner shall verify that the listed references remain current and update the manual where requirements have changed.